SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE CODE:  CSA0713                    COURSE NAME: COMPUTER NETWORKS

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

Code of Conduct:

I MohamedFasilHak.M(192524304) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Mohamed Fasil Hak.M

Annexure A

ANALYTICAL PROBLEM SOLVING

1. Suppose we want to transmit the message 11100011 and protect it from errors using the CRC polynomial X3+1.

i.) Use polynomial long division to determine the message that should be transmitted.

ii) Suppose the leftmost bit of the message is inverted due to noise on the transmission link. What is the result of the receiver's CRC calculation? How does the receiver know that an error has occurred?

Given

- Message = 11100011

- Generator Polynomial = X3+1

- Generator Binary = 1001

Step 1: Append Zeros

Since the generator degree is 3, append 3 zeros to the message.

Original Message : 11100011

Appended Message : 11100011000

Step 2: Perform Modulo-2 (XOR) Division

Generator = 1001

_________________

1001 ) 11100011000

1001

----------

1110

1001

----------

1111

1001

----------

1111

1001

----------

1101

1001

----------

1000

1001

----------

00100

CRC Remainder = 100

Original Message = 11100011

CRC             = 100

--------------------------------

Transmitted Data = 11100011100

Error Detection

If the leftmost bit is inverted due to noise,

Received Data = 01100011100

The receiver performs CRC division again using the same generator (1001).

Since the remainder is non-zero, the received data is not divisible by the generator polynomial.

Final Answer

- Generator = 1001

- CRC Remainder = 100

- Transmitted Message = 11100011100

- After the first bit is inverted, the receiver obtains a non-zero remainder and detects an error.

2. a) A network with bandwidth of 10 Mbps can pass only an average of 12,000 frames per minute with each frame carrying an average of 10,000 bits. What is the throughput of this network?

Given

- Bandwidth = 10 Mbps

- Frames transmitted = 12,000 frames/min

- Frame size = 10,000 bits

Method

Throughput is the actual amount of data transmitted per second.

Throughput=Frames×Frame SizeTime

Calculation

Total bits transmitted in one minute:

12000×10000=120,000,000 bits

Convert to bits per second:

120,000,00060=2,000,000 bps
=2 Mbps

Network Efficiency:

210×100=20%

Final Answer

- Throughput = 2 Mbps

- Network Efficiency = 20%

b) A network has a bandwidth of 3000 Hz (300 to 3300 Hz) assigned for data communications. The signal- to-noise ratio is 3162. Find the channel capacity

Given

- Bandwidth, B = 3000 Hz

- Signal-to-Noise Ratio, SNR = 3162

Method

Using Shannon Capacity Formula:

C=Blog21+SNR

Calculation

C=3000×log23163

Since,

log23163≈11.63
C=3000×11.63
C=34,890 bps
C≈34.9 kbps

Final Answer

Channel Capacity = 34.9 kbps

3. A multispecialty hospital uses wireless patient-monitoring devices to continuously transmit patients' heart rate and blood pressure readings to a central monitoring station. Since the data is transmitted over a wireless channel, electromagnetic interference occasionally changes one bit during transmission. To ensure that doctors receive accurate patient information without waiting for retransmission, the hospital implements the Hamming (12,8) error-correcting code.

Code Structure

With m = 8 data bits, the number of redundancy (parity) bits r must satisfy: 2^r ≥ m + r + 1. r = 4 satisfies this (2^4 = 16 ≥ 13), giving a total codeword length n = m + r = 12 bits — hence Hamming (12,8).

| Position | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit Type | P1 | P2 | D1 | P4 | D2 | D3 | D4 | P8 | D5 | D6 | D7 | D8 |

Parity bits are placed at power-of-2 positions (1, 2, 4, 8); data bits fill the remaining positions.

Parity Bit Coverage

- P1 (position 1): covers all positions whose binary index has bit-0 set → 1, 3, 5, 7, 9, 11

- P2 (position 2): covers all positions whose binary index has bit-1 set → 2, 3, 6, 7, 10, 11

- P4 (position 4): covers all positions whose binary index has bit-2 set → 4, 5, 6, 7, 12

- P8 (position 8): covers all positions whose binary index has bit-3 set → 8, 9, 10, 11, 12

Each parity bit is set (0 or 1) so that the total number of 1s in its coverage group is even (even parity).

Example Encoding

Let the 8-bit patient data be: 1 0 1 1 0 1 0 1, placed into D1–D8 respectively. Each parity bit is computed as the XOR of the data bits it covers, ensuring even parity in each group. This produces the final 12-bit codeword that is transmitted over the wireless channel.

Error Detection and Correction at the Receiver

When the 12-bit codeword arrives, the receiver recomputes each parity check and XORs the received parity bit with the recalculated value, forming a 4-bit binary syndrome (S8 S4 S2 S1):

- If the syndrome = 0000 → no error occurred; data accepted as-is.

- If the syndrome is non-zero → its decimal value directly gives the exact bit position that is in error (this is the key property of Hamming codes).

- The receiver flips that exact bit, correcting the single-bit error instantly.

4. a) A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

Given

- Network Address = 172.16.10.0/24

- Subnet Mask = /27

- IP Address 1 = 172.16.10.78

- IP Address 2 = 172.16.10.95

The question asks to determine:

- Total number of subnets

- Usable hosts per subnet

- Subnet of 172.16.10.78

- Network and Broadcast addresses

- Whether 172.16.10.95 belongs to the same subnet

Method

1. Total Number of Subnets

Original Prefix = /24

New Prefix = /27

Borrowed Bits

27-24=3

Number of Subnets

23=8

Total Subnets = 8

2. Usable Hosts per Subnet

Host Bits

32-27=5

Total Addresses

25=32

Usable Hosts

32-2=30

Usable Hosts per Subnet = 30

3. List of /27 Subnets

| Subnet | Address Range | Broadcast |
| --- | --- | --- |
| 1 | 172.16.10.0 – 31 | 172.16.10.31 |
| 2 | 172.16.10.32 – 63 | 172.16.10.63 |
| 3 | 172.16.10.64 – 95 | 172.16.10.95 |
| 4 | 172.16.10.96 – 127 | 172.16.10.127 |
| 5 | 172.16.10.128 – 159 | 172.16.10.159 |
| 6 | 172.16.10.160 – 191 | 172.16.10.191 |
| 7 | 172.16.10.192 – 223 | 172.16.10.223 |
| 8 | 172.16.10.224 – 255 | 172.16.10.255 |

4. Subnet for 172.16.10.78

Since

64≤78≤95

Subnet = 172.16.10.64/27

5. Network Address

172.16.10.64

6. Broadcast Address

172.16.10.95

7. Does 172.16.10.95 belong to the same subnet?

Yes.

However,

- 172.16.10.95 is the Broadcast Address of the subnet.

- It is not a usable host address.

- It still belongs to the same subnet, but it is reserved for broadcasting.

Final Answer

- Subnet Mask = /27 (255.255.255.224)

- Total Subnets = 8

- Usable Hosts per Subnet = 30

- 172.16.10.78 belongs to subnet 172.16.10.64/27

- Network Address = 172.16.10.64

- Broadcast Address = 172.16.10.95

- 172.16.10.95 belongs to the same subnet, but it is the broadcast address and cannot be assigned to a host.

b) A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

Given

- IPv6 Address:
2001:0db8:0000:0000:0000:ff00:0042:8329

- Prefix Length = /64

The question asks to:

- Compress the IPv6 address.

- Expand it back to the original form.

- Determine the network prefix.

- Calculate the total number of host addresses.

Method

1. Compress the IPv6 Address

Rules:

- Remove leading zeros from each block.

- Replace one continuous sequence of all-zero blocks with ::.

Original Address:

2001:0db8:0000:0000:0000:ff00:0042:8329

Compressed Address:

2001:db8::ff00:42:8329

2. Expand the IPv6 Address

Expand the compressed form by restoring omitted zero blocks and leading zeros.

Expanded Address:

2001:0db8:0000:0000:0000:ff00:0042:8329

3. Determine the Network Prefix (/64)

A /64 prefix means the first 64 bits (first four hexadecimal blocks) represent the network portion.

Network Prefix:

2001:0db8:0000:0000::/64

4. Calculate the Number of Host Addresses

Host Bits:

128-64=64

Total Host Addresses:

264
=18,446,744,073,709,551,616

≈ 18.4 Quintillion Host Addresses

Final Answer

| Parameter | Answer |
| --- | --- |
| Compressed IPv6 Address | 2001:db8::ff00:42:8329 |
| Expanded IPv6 Address | 2001:0db8:0000:0000:0000:ff00:0042:8329 |
| Network Prefix | 2001:0db8:0000:0000::/64 |
| Host Bits | 64 |
| Total Host Addresses | 2⁶⁴ = 18,446,744,073,709,551,616 |

5. A university is conducting an online semester examination for 8,000 students simultaneously. At exactly 10:00 AM, the examination server begins transmitting encrypted question papers to all students. The university server is connected through a 10 Gbps high-speed network, while students access the examination portal using different internet connections such as fiber broadband (100 Mbps), home Wi-Fi (20 Mbps), and mobile networks (5–10 Mbps). During the first few minutes of the examination, many students with slower internet connections report incomplete downloads, repeated retransmissions, delayed acknowledgments, and temporary freezing of the examination portal. Network administrators observe that the server continues transmitting data at a high rate even though several student devices are unable to receive and process packets at the same speed. This results in receiver buffer overflow, unnecessary retransmissions, increased network traffic, and delays in delivering the question papers. Analyze the above scenario and identify the primary networking problem. Explain how the mismatch between the sender's transmission speed and the receiver's processing capability affects communitcation

Given

- University server speed = 10 Gbps

- Students = 8,000

- Student Internet Speeds:

  - Fiber Broadband = 100 Mbps

  - Home Wi-Fi = 20 Mbps

  - Mobile Network = 5–10 Mbps

- Observed Problems:

  - Incomplete downloads

  - Repeated retransmissions

  - Delayed acknowledgments

  - Buffer overflow

  - Increased network traffic

  - Temporary freezing of the examination portal

Problem Identification

The primary networking problem is Flow Control.

The sender (server) transmits data much faster than some receivers (student devices) can receive and process.

Analysis

Sender Speed

10 Gbps

Receiver Speeds

Fiber Broadband : 100 Mbps

Home Wi-Fi      : 20 Mbps

Mobile Network  : 5–10 Mbps

Since the server transmits at 10 Gbps, many receivers cannot process incoming packets at the same rate.

Effects of Speed Mismatch

- Receiver buffers become full (Buffer Overflow).

- Packets are dropped.

- Lost packets trigger retransmissions.

- Acknowledgments are delayed.

- Network congestion increases.

- Downloads become incomplete or freeze.

- Overall communication efficiency decreases.

Solution

The sender should implement Flow Control mechanisms such as:

- Sliding Window Protocol

- TCP Flow Control

- Receiver Window Advertisement

These mechanisms allow the sender to adjust its transmission rate according to the receiver's processing capability.

Final Answer

- Primary Networking Problem: Flow Control

- Cause: Sender transmits data faster than receivers can process.

- Effects:

  - Buffer overflow

  - Packet loss

  - Retransmissions

  - Delayed acknowledgments

  - Network congestion

  - Slow or incomplete downloads

- Solution: Use TCP Flow Control or Sliding Window Protocol to match the sender's transmission rate with the receiver's capacity.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |