SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO5: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL5)

Assessment Tool Weightage: 40%

QUESTIONS: 3								Total Marks:30

Annexure A

Constraint-Based Problem Solving

Code of Conduct:

Imohamed Fasil Hak.M/192524304(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Mohamed Fasil Hak.M

Annexure A

Constraint-Based Problem Solving

- A  multinational company is opening a new branch office in a remote rural location where high-speed wired internet is unavailable. The office must support cloud-based applications, video conferencing, and secure communication with the headquarters. The company has a limited budget for connectivity but requires uninterrupted business operations. As a network engineer, recommend a suitable internet connection, networking devices, and security mechanisms. Justify your recommendations by considering bandwidth limitations, reliability, cost, and future scalability

- A university plans to provide campus-wide wireless internet access for more than 3,000 students across classrooms, laboratories, libraries, and hostels. The network must support a large number of simultaneous users while minimizing signal interference and ensuring secure access. Due to budget constraints, only a limited number of wireless access points can be deployed. Analyze the requirements and propose an efficient wireless network design, including access point placement, frequency band selection, and authentication methods. Explain how your solution balances coverage, performance, security, and cost.

- A financial institution is upgrading its internal computer network to improve cybersecurity while allowing employees to access sensitive customer information and online banking services. The organization must comply with strict security policies without significantly affecting network performance or increasing operational costs. Design a secure network architecture by selecting appropriate network segmentation techniques, firewall deployment, authentication methods, and intrusion detection mechanisms. Justify your design by considering the constraints of security, performance, compliance, and budget.

RUBRICS FOR CALCULATION

| Criteria | Weightage (%) | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Problem & Constraints | 20% | Clearly identifies all constraints and fully understands the problem requirements | Identifies most constraints with minor gaps in understanding | Identifies some constraints but misses key aspects | Shows little or no understanding of constraints or problem |
| Problem Decomposition | 15% | Breaks problem into logical, well-structured sub-problems effectively | Breaks problem into sub-parts with some logical flow | Attempts decomposition but lacks clarity or structure | No clear decomposition of the problem |
| Application of Constraints in Solution | 20% | Applies all constraints correctly and consistently in solution design | Applies most constraints correctly with minor errors | Applies some constraints but with noticeable errors | Fails to apply constraints appropriately |
| Logical Reasoning & Strategy | 15% | Uses strong, clear, and efficient reasoning strategies | Uses generally sound reasoning with minor gaps | Reasoning is partially correct but inconsistent | Reasoning is unclear or incorrect |
| Solution Accuracy & Feasibility | 15% | Solution is fully correct, feasible, and optimized | Solution is mostly correct and feasible | Solution has errors or is partially feasible | Solution is incorrect or not feasible |
| Creativity & Innovation | 5% | Demonstrates highly innovative and unique approach | Shows some creativity in approach | Limited creativity shown | No creativity; relies on basic or copied ideas |
| Clarity of Presentation | 10% | Solution is clearly presented with excellent organization and explanation | Presentation is clear with minor issues | Presentation lacks clarity or organization | Poorly presented and difficult to understand |

1. A  multinational company is opening a new branch office in a remote rural location where high-speed wired internet is unavailable. The office must support cloud-based applications, video conferencing, and secure communication with the headquarters. The company has a limited budget for connectivity but requires uninterrupted business operations. As a network engineer, recommend a suitable internet connection, networking devices, and security mechanisms. Justify your recommendations by considering bandwidth limitations, reliability, cost, and future scalability

Recommended solution:

Use a 4G/5G fixed wireless connection as the primary internet connection, with a satellite connection as backup if mobile coverage is unreliable.

1. Internet Connection

- Primary: 4G/5G Fixed Wireless Access (FWA).

- Backup: Satellite internet for emergency connectivity.

- FWA is suitable because it provides good bandwidth without requiring wired infrastructure.

- Satellite backup improves reliability and helps maintain business operations during mobile-network outages.

2. Networking Devices

- 5G/4G Router: Connects the office to the internet.

- Firewall: Protects the internal network from unauthorized access.

- Managed Switch: Connects computers, printers, IP phones, and other wired devices.

- Wi-Fi Access Points: Provide wireless connectivity throughout the office.

- VPN Router/Gateway: Creates a secure connection to company headquarters.

- UPS: Keeps networking equipment running during short power failures.

3. Security Mechanisms

- Use a next-generation firewall with intrusion prevention.

- Establish an IPsec/SSL VPN between the branch and headquarters.

- Use WPA3 for Wi-Fi security.

- Enable strong passwords and multi-factor authentication (MFA).

- Keep routers, firewalls, and access points updated with security patches.

- Separate employees, guests, and critical systems using VLANs.

4. Constraint-Based Justification

| Constraint | Recommendation | Justification |
| --- | --- | --- |
| Bandwidth | 4G/5G FWA | Provides sufficient bandwidth for cloud applications and video meetings |
| Reliability | Satellite backup | Provides an alternative connection if FWA fails |
| Cost | FWA as primary | Less expensive than dedicated leased lines in remote areas |
| Security | Firewall + VPN + WPA3 | Protects business data and communication |
| Scalability | Managed switch + additional APs | Allows more users and devices to be added later |
| Uninterrupted operation | Dual WAN + UPS | Maintains connectivity during network or power interruptions |

Conclusion

The best solution is a 4G/5G FWA connection with satellite backup, supported by a router, firewall, managed switch, Wi-Fi access points, VPN, and UPS. This provides a good balance of bandwidth, reliability, security, affordability, and future scalability for a rural branch office.

2. A university plans to provide campus-wide wireless internet access for more than 3,000 students across classrooms, laboratories, libraries, and hostels. The network must support a large number of simultaneous users while minimizing signal interference and ensuring secure access. Due to budget constraints, only a limited number of wireless access points can be deployed. Analyze the requirements and propose an efficient wireless network design, including access point placement, frequency band selection, and authentication methods. Explain how your solution balances coverage, performance, security, and cost.

Recommended Wireless Network Design

For a university with more than 3,000 students, I recommend a centrally managed Wi-Fi network using carefully positioned dual-band Wi-Fi 6 access points.

1. Access Point Placement

- Install APs in classrooms, laboratories, libraries, hostels, and common areas.

- Place APs at central locations such as ceilings or corridors to provide better coverage.

- Use a site survey before installation to identify dead zones and interference.

- Give higher AP density to high-user areas such as libraries and lecture halls.

- Avoid unnecessary APs because the budget is limited.

2. Frequency Band Selection

| Band | Usage | Reason |
| --- | --- | --- |
| 2.4 GHz | Basic/low-bandwidth devices | Longer range but more interference |
| 5 GHz | Main student devices | Higher speed and more available channels |
| 6 GHz | Newer compatible devices | High capacity and lower congestion where supported |

- Use 5 GHz as the primary band for students because it provides better performance.

- Use 2.4 GHz mainly for devices that require greater range or do not support 5 GHz.

- If the university's equipment and regulations support it, 6 GHz can be used for newer devices.

- Configure channels carefully to reduce co-channel interference and avoid overlapping channels where possible.

3. Authentication and Security

Use WPA3-Enterprise with 802.1X authentication.

- A RADIUS server authenticates students using their university credentials.

- Each user receives individual authentication rather than a shared Wi-Fi password.

- Create separate networks/VLANs for:

  - Students

  - Faculty and staff

  - Guests

  - University IoT devices

- Enable firewall rules and access controls between VLANs.

4. Handling 3,000+ Simultaneous Users

- Use Wi-Fi 6 APs supporting high client density.

- Use band steering to move capable devices toward 5 GHz/6 GHz.

- Enable load balancing so users are distributed across APs.

- Use QoS to prioritize important applications such as online classes and video conferencing.

- Use a wireless LAN controller/cloud controller for centralized monitoring and configuration.

5. Balancing the Constraints

| Requirement | Solution | Benefit |
| --- | --- | --- |
| Large number of users | Wi-Fi 6 + load balancing | Supports high client density |
| Limited AP budget | Site survey + strategic placement | Avoids unnecessary APs |
| Interference | Channel planning + 5/6 GHz | Improves performance |
| Coverage | Ceiling-mounted APs | Reduces dead zones |
| Security | WPA3-Enterprise + 802.1X | Secure individual authentication |
| Performance | Band steering + QoS | Better user experience |
| Scalability | Central controller + VLANs | Easy future expansion |

Conclusion

The most efficient solution is a centrally managed Wi-Fi 6 network with strategically placed dual-band APs, using 5 GHz as the primary student-access band and 2.4 GHz for longer-range/legacy devices. WPA3-Enterprise with 802.1X/RADIUS provides secure access. Site surveying, channel planning, band steering, and load balancing allow the university to support 3,000+ students with a limited number of APs, while maintaining a good balance between coverage, performance, security, and cost.

3.A financial institution is upgrading its internal computer network to improve  cybersecurity while allowing employees to access sensitive customer information and online banking services. The organization must comply with strict security policies without significantly affecting network performance or increasing operational costs. Design a secure network architecture by selecting appropriate network segmentation techniques, firewall deployment, authentication methods, and intrusion detection mechanisms. Justify your design by considering the constraints of security, performance, compliance, and budget.

Recommended Secure Network Architecture

For a financial institution, I recommend a layered security architecture using VLAN segmentation, next-generation firewalls, multi-factor authentication, and IDS/IPS. This protects sensitive customer data while maintaining good network performance and controlling costs.

1. Network Segmentation

Divide the network into separate VLANs/security zones:

- Employee VLAN – normal office users.

- Customer Data VLAN – highly sensitive customer information.

- Banking/Application VLAN – online banking and critical applications.

- Server VLAN/DMZ – publicly accessible services.

- Guest VLAN – internet-only access for visitors.

- Management VLAN – network devices and administrator access.

Use ACLs (Access Control Lists) between VLANs so employees can access only the resources required for their jobs.

2. Firewall Deployment

Deploy a Next-Generation Firewall (NGFW) at the internet gateway.

- Controls incoming and outgoing traffic.

- Blocks unauthorized connections.

- Provides application and user-based filtering.

- Performs intrusion prevention.

- Use a DMZ for public-facing services such as web servers.

- For high availability, two firewalls can operate in an active/standby configuration.

3. Authentication

Use strong authentication methods:

- MFA (Multi-Factor Authentication) for employees and administrators.

- 802.1X authentication for network access.

- A centralized RADIUS/AAA server for authentication and authorization.

- Use role-based access control (RBAC) so employees receive only the permissions required for their job.

- Privileged administrator accounts should have stronger authentication and restricted access.

4. Intrusion Detection and Prevention

Deploy IDS/IPS at important network points.

- IDS monitors traffic and generates alerts for suspicious activity.

- IPS can automatically block known malicious traffic.

- Monitor traffic between critical VLANs as well as internet traffic.

- Send security logs to a centralized SIEM system for monitoring and auditing.

5. Balancing the Constraints

| Constraint | Solution | Benefit |
| --- | --- | --- |
| Security | VLANs + ACLs + NGFW | Limits unauthorized access |
| Performance | Hardware-based firewall + selective inspection | Reduces unnecessary processing |
| Compliance | MFA + logging + RBAC | Supports auditing and security policies |
| Budget | Use existing switches with VLANs | Avoids replacing all network equipment |
| Sensitive data | Separate customer-data VLAN | Provides stronger isolation |
| Threat detection | IDS/IPS | Detects and blocks suspicious activity |
| Scalability | Centralized authentication and monitoring | Easy to expand |

Simple Architecture

Internet → NGFW → DMZ / Core Switch → VLANs

- VLAN 10 → Employees

- VLAN 20 → Customer Data

- VLAN 30 → Banking Applications

- VLAN 40 → Servers

- VLAN 50 → Guest Network

- VLAN 60 → Network Management

IDS/IPS + SIEM monitor important traffic and security events.

Conclusion

The proposed architecture uses network segmentation, NGFW, MFA, 802.1X, RBAC, and IDS/IPS to provide layered protection. VLANs and ACLs isolate sensitive customer information, while the firewall protects the organization from external threats. Centralized authentication and logging support compliance requirements. Using existing managed switches for VLAN segmentation and deploying security controls selectively helps control operational costs without significantly reducing network performance.